Radiography Concepts	 			Name:____________________________

Score:

LAB #5: OID Vs. Magnification and Sharpness

Materials needed: A resolution test template; A small, dry bone, such as a phalanx; A smooth-edged coin, and several flat, stackable sponges to total at least 8 inches of thickness which can be precisely divided in half.

Procedure

Place a 10" X 12" (25 X 30 cm) CR cassette on the tabletop, centered to the x-ray beam. Collimate the field to about 1 inch (2.5 cm) smaller than the cassette on all sides, (and do not change collimation throughout the experiment). Then stack sponges to create an OID of 4 - 6 inches (10 - 15 cm). Place a small dry bone and a smooth-edged coin very close to the central ray on top of the sponges. Place the resolution template alongside the coin and bone, crosswise to the table so that the lines in it are lengthwise with the table.

Be sure to measure SID from the tabletop, not from the bucky. The OID will be changed as indicated below. Label your exposures with lead markers to keep track of them.

Set a fixed kVp to 46 kVp

Exposure #1: 	  2 mAs,  40" (100 cm) SID, no OID - place objects directly on the cassette

Exposure #2: 	  2 mAs,  40" (100 cm) SID, 4 - 6" (10 - 15 cm) OID

Exposure #3: 	  2 mAs,  40" (100 cm) SID, 8 - 12" (20 - 30 cm) OID, exactly 2 X OID

Analysis

1. Accurately measure the width of the actual coin in mm. Carefully measure the width of each 	of the images of the coin. Record these measurements below:

Actual coin   =                  mm

Exposure #1 coin image   =                  mm

Exposure #2 coin image   =                  mm

Exposure #3 coin image   =                  mm

2. Calculate the ratio of magnification for each image by dividing the real coin width into the

width of each image:

Exposure #1   (no OID) magnification ratio   =      Image      =                    =

Object

Exposure #2 (short OID) magnification ratio =      Image      =                    =

Object

Exposure #3  (long OID) magnification ratio =      Image      =                    =

Object

3. Compare the magnification ratios in #2; As the object-image distance is increased, what

happens to the size of the image (if anything)?

4. Considering these results, what general rule can you state for OID in practice to keep

magnification under control?

5. Compare the shape of the coin images to the real coin shape; Do changes in OID affect shape

distortion in the image?

6. On each image of the resolution template, begin with the thickest set of lines and scan

downward through the sets until you first lose resolution of the lines. Have a partner

check to see if they agree where this point is. [Note that if you go beyond the first

complete blurring of the lines, they may seem to re-appear - this is false resolution, in

which each line blurs so much that it overlaps the next line. You must use the first blur

point.] Go to the previous set of lines - the last set in which the lines can be clearly

distinguished - and read the line-pair per millimeter number to the side. This is the

resolution. Record below:

Exposure #1    (no OID) sharpness   =              LP/mm

Exposure #2 (short OID) sharpness   =              LP/mm

Exposure #2  (long OID) sharpness   =              LP/mm

7. Which SID produced the sharpest image?

8. Examine the trabeculae in the marrow of the small bone. Can you visually see any substantial

difference between the sharpness of these details? If so, explain:

9. Considering these results, what general rule can you state for SID in practice to keep sharpness

as high as possible?