Radiography Concepts	 			Name:_____________________________

Score:

LAB #6: Distance Ratios Vs. Magnification and Sharpness

Materials needed: A resolution test template; A small, dry bone, such as a phalanx; A smooth-edged coin, and several flat, stackable sponges to total at least 8 inches of thickness which can be precisely divided in half.

Procedure

Place a 10" X 12" (25 X 30 cm) CR cassette on the tabletop, centered to the x-ray beam. Collimate the field to about 1 inch (2.5 cm) smaller than the cassette on all sides, (and do not change collimation throughout the experiment). Then stack sponges to create an OID of 8 - 12 inches (20 - 30 cm). Place a small dry bone and a smooth-edged coin very close to the central ray on top of the sponges. Place the resolution template alongside the coin and bone, crosswise to the table so that the lines in it are lengthwise with the table.

Be sure to measure SID from the tabletop, not from the bucky. You may need a tape measure to check for a 20-inch (50 cm) SID - Measure from the tabletop to the “X” or “+” etched on the anode-end of the x-ray tube housing. Both the SID and the OID will be changed as indicated below. Label your exposures with lead markers to keep track of them.

Set a fixed kVp to 46 kVp

Exposure #1: 	  2 mAs,  40" (100 cm) SID, 8 - 12" (20 - 30 cm) OID

Exposure #2:  0.6 mAs, 20" (50 cm) SID, 4 - 6" (10 - 15 cm) OID, exactly ½ SID & OID

Analysis

1. Accurately measure the width of the actual coin in mm. Carefully measure the width of each of

the images of the coin. Record these measurements below:

Actual coin   =                  mm

Exposure #1 coin image   =                  mm

Exposure #2 coin image   =                  mm

2. Magnification can be predicted by dividing the SID by the source-object distance (SOD).

(Don’t forget to find the SOD by subtracting the OID from the SID.) Calculate the

predicted magnification ratio for each projection and record below:

Exposure #1 (long distances) magnification ratio  =      SID      =                   =   _______

SOD

Exposure #2 (short distances) magnification ratio  =      SID      =                   =                 							   		    SOD

3. Compare the magnification ratios in #2; What do you notice about these?

4. From #1, calculate the actual magnification ratio for each image by dividing the real coin

width into the width of each image:

Exposure #1 (long distances) magnification ratio  =      Image      =                   =  _______

Object

Exposure #2 (short distances) magnification ratio  =      Image      =                   =   ______              							                Object

5. Compare the magnification ratios in #4; What do you notice about these? In terms of distance

ratios, why did you get these results?

6. A relative sharpness value can be predicted by dividing the SOD by the OID. Calculate the

predicted relative sharpness for each projection and record below:

Exposure #1 (long distances) magnification ratio  =      SOD      =                   =   _______

OID

Exposure #2 (short distances) magnification ratio  =      SOD      =                   =                 									     OID

7. Compare the magnification ratios in #2; What do you notice about these?

8. On each image of the resolution template, begin with the thickest set of lines and scan

downward through the sets until you first lose resolution of the lines. Have a partner

check to see if they agree where this point is. [Note that if you go beyond the first

complete blurring of the lines, they may seem to re-appear - this is false resolution, in

which each line blurs so much that it overlaps the next line. You must use the first blur

point.] Go to the previous set of lines - the last set in which the lines can be clearly

distinguished - and read the line-pair per millimeter number to the side. This is the

resolution. Record below:

Exposure #1  (long distances) sharpness   =              LP/mm

Exposure #2  (short distances) sharpness   =              LP/mm

7. Is there any difference in sharpness between the two sets of distances used? In terms of

distance ratios, why did you get these results?

8. Examine the trabeculae in the marrow of the small bone. Can you visually see any substantial

difference between the sharpness of these details? If so, explain:

9. When considering the control of either sharpness or magnification in radiographic projections,

can the SID, SOD or OID be considered separately? Why or why not?