Radiography Concepts	 			Name:___________________________

Score:

LAB #1: Adequate Penetration and kVp

Materials needed: A tissue-equivalent (plexiglass) pelvis/abdomen phantom. Lead markers.

Procedure

Place a 14" X 17" (35 X 42 cm) CR cassette in the bucky. Center a pelvis/abdomen phantom to the bucky. With a 40" or 100 cm SID, center the x-ray beam and collimate to fill the cassette plate. Do not change collimation, centering or distance throughout the experiment.

The kVp and mAs will be changed as indicated below, with increases in mAs attempting to compensate for a low-penetration 40-kVp exposure. Label your exposures with lead markers to keep track of them. Print out the images with the instructor’s permission for side-by-side comparison.

Techniques:    	Exposure #1:	80 kVp, 10 - 12 mAs (or as indicated by the instructor)

Exposure #2:	40 kVp, 25 mAs	(½ kVp, 2 X mAs)

Exposure #2:	40 kVp, 50 mAs	(½ kVp, 4 X mAs)

Exposure #2:	40 kVp, 100 mAs	(½ kVp, 8 X mAs)

Analysis

1. On Exposure #1, has a satisfactory darkness level (density) been achieved in the acetabulum

and other dense bone areas? (If not, repeat with a higher kVp for this reference image.)

2. On the remaining exposures, examine particularly the dense bone areas such as the acetabulum

area. Slight increases in density may be seen, but using Exposure #1 for comparison, is a

satisfactory demonstration of the details within the bones in these areas ever achieved

with the increasing mAs values?

3. Do you believe that a satisfactory demonstration of details in these areas would be achieved if

the mAs were increased to 500?

4. Can increasing x-ray beam intensity ever compensate for inadequate penetration?

5. Reword the principle of radiographic technique in #4 in terms of mAs and kVp: