Radiation Protection					Name:______________________________

Score:

LAB #1: Dose Rate and Integrated Dose Vs. mA

Materials needed: Ion chamber, lead aprons.

In the radiography laboratory, set up the ion chamber on the x-ray table centered under the x-ray tube at 40 inches (100 cm) SID. Set the following techniques and record the readings from the ion chamber, re-setting it properly for each type of measurement. USE PROPER UNITS IN REPORTING EACH MEASUREMENT.

*NOTE: For RATE measurements, you will have to put on a lead apron or stand directly behind someone who has a lead apron on, and get up close to the meter. During the 2-second exposure, the meter will read out several “rate” measurements. You must try to mentally track all of these readings and estimate a reasonable average between them. You may repeat the exposure to obtain additional readings.

EXPOSURE		     INTEGRATED

RATE      		TOTAL EXPOSURE

1.  60 kVp, 2 seconds, 50 mA:

2.  60 kVp, 2 seconds, 100 mA:

1. At double the mA setting, how much did the measured exposure RATE change?

2. Is this what you should expect? How accurate was this? If it was off, which way and how far (what percent)?

3. At double the mA setting, how much did the measured total integrated exposure change?

4. Is this what you should expect? How accurate was this? If it was off, which way and how far (what percent)?