Computerized Radiography (CR) 			Name:_____________________________

Score:

LAB #7: Resilience of Digital Processing to Scatter Radiation Caused During Exposure

Materials needed: Tissue-equivalent abdomen phantom (large dense phantom to generate scatter)

*If a film/screen system with processing is available, Part B can be included

for a dramatic comparison of digital systems to film/screen systems for scatter sensitivity.

Procedure

To maximize the amount of scatter radiation produced, take all exposures non-grid and table-top. Place an abdomen phantom directly on a 14 X 17" (35 X 43 cm) CR plate on the x-ray table. Open the collimated field to fill the receptor plate. Do not change collimation, distance, or centering between exposures. Be sure to turn off the AEC and use “manual techniques.”

To eliminate any background exposure, erase all CR cassettes in the CR processor prior to using them. Tape lead markers to designate the cassettes for Exposure #1, #2 and #3.

With the instructor’s permission, print out the images for side-by-side comparison on a viewbox.

PART A:

Exposures:

Exposure #1: Set a technique of 80 kVp at 10 mAs.

Exposure #2: 120 kVp at 2 mAs

Exposure #3: 120 kVp at 4 mAs

Exposure #4: 120 kVp at 8 mAs

PART B: If a film/screen system is available, repeat all four of the above exposures using this

system.

Analysis: PART A

1. Observe all four images. Is any fog density visible on any of these views? Do any of them manifest a significant loss of contrast?

2. If none of these views showed any fog density, why not? (See textbook)

3. If only Exposure #4 demonstrated a significant loss of contrast, explain how this occurs in a

digitally processed image (See textbook):

4. Based on these overall results, how effective would you say digital processing is at “cleaning

up” the effects of scatter radiation caused during exposure?

Analysis: PART B

1. Observe all four of the film/screen images. Is any fog density visible on any of these views? Do any of them manifest a significant loss of contrast? If so, which ones?

2. Compare this entire series with the corresponding radiographs in the digitally-processed series.

Based on these overall results, how effective would you say digital processing is at

“cleaning up” the effects of scatter radiation caused during exposure, when compared to

film/screen processing?